GuardX

Predictive Maintenance System for CNC Machines

APP FLOW DOCUMENT

Version 1.0 — MVP (Hackathon Build)

TN Impact 2026 | TNI26026

| Product | GuardX – Open-Source CNC Predictive Maintenance |
| --- | --- |
| Version | 1.0 (MVP) |
| Type | IoT + ML Hybrid System |
| Target Users | SME Factory Owners, Maintenance Engineers, Operators |
| Problem Statement | TNI26026 – CNC Predictive Maintenance via Open-Source Tools |
| Event | TN Impact 2026, Sona College of Technology — 28 Feb 2026 |

# 1. Product Overview

GuardX is a low-cost, open-source predictive maintenance framework designed for SMEs operating CNC machines. It combines IoT-enabled real-time sensor data acquisition with an evolving machine learning pipeline — starting with unsupervised anomaly detection and progressively transitioning to supervised failure classification as labeled data grows.

The system democratizes Industry 4.0 capabilities, making advanced predictive maintenance accessible without proprietary software or expensive hardware.

## 1.1 Core Problem

- CNC machine failures cause unplanned downtime — costing ₹20,000–₹50,000+ per incident

- Most SMEs use reactive maintenance (repair after failure) due to cost constraints

- Commercial predictive maintenance tools are expensive and not SME-friendly

- There is no affordable, modular, open-source alternative tailored for SMEs

## 1.2 GuardX Solution Summary

- Real-time sensor monitoring via IoT (vibration, temperature, current)

- Intelligent anomaly detection using Isolation Forest (unsupervised)

- Progressive ML evolution to supervised failure classification

- Machine Health Score (0–100) for non-technical operators

- Open-source, modular, on-premise or cloud deployable

## 1.3 Target Users

| User Type | Primary Need |
| --- | --- |
| SME Factory Owner | Reduce breakdown costs, maximize uptime |
| Maintenance Engineer | Early warning, scheduled intervention |
| Production Supervisor | Real-time machine health visibility |
| IoT/ML Integrators | Modular extensible architecture |

# 2. System Architecture

GuardX is built on an 8-layer modular architecture. Each layer has a specific responsibility and outputs a well-defined artifact that feeds into the next layer. This separation enables maintainability, scalability, and independent testing of each component.

GuardX — Complete Data Flow

Physical Layer → Edge → Ingestion → Preprocessing → Storage → ML Engine → Health Engine → Dashboard

| # | Layer Name | Purpose | Output |
| --- | --- | --- | --- |
| L1 | Physical Machine Layer | Motor + sensors generate real mechanical signals (vibration, temperature, load) | Raw physical signals |
| L2 | Edge Acquisition Layer | ESP32 reads sensors, timestamps data, sends JSON via WiFi | JSON stream: {timestamp, vibration, temperature, current} |
| L3 | Data Ingestion Layer | FastAPI backend receives JSON, validates, logs raw data | Raw time-series table in database |
| L4 | Preprocessing Layer | Moving average, normalization, feature extraction (RMS, gradient, variance) | Structured ML-ready feature vectors |
| L5 | Time-Series Storage | Stores raw + processed data, anomaly scores, labels | Queryable historical time-series DB |
| L6 | ML Intelligence Engine | Isolation Forest → Random Forest → XGBoost (evolving model pipeline) | Anomaly score + failure class + probability |
| L7 | Health Score Engine | Converts ML output to Health Score 0–100 with risk classification | Health Score + Risk Level (Healthy/Warning/Critical/High Risk) |
| L8 | Alert & Visualization | Streamlit dashboard with live graphs, gauges, alerts, notifications | Operator-facing insights + triggered alerts |

# 3. Detailed Layer-by-Layer App Flow

## Layer 1 — Physical Machine Layer

This is the signal source layer. A 12V DC geared motor acts as the CNC spindle proxy, with low-cost sensors attached to capture real electromechanical behavior.

Components

• 12V DC Geared Motor (300–600 RPM) — CNC spindle proxy

• MPU6050 Accelerometer/Gyroscope — Captures 3-axis vibration

• DS18B20 Temperature Sensor — Measures motor casing heat

• ACS712 Current Sensor — Measures load (spindle load proxy)

• Offset weight (nut/washer) — Simulates mechanical imbalance

• Wooden baseboard — Firm mechanical mounting

Simulation Scenarios

• Scenario 1 — Normal Operation: Balanced motor, stable vibration baseline

• Scenario 2 — Imbalance Fault: Small asymmetric weight on shaft causes vibration spikes

• Scenario 3 — Load Increase: Fan blade / resistance applied; current and temperature rise

• Scenario 4 — Degradation Pattern: Sustained imbalance run showing gradual vibration trend

Drawback: Hardware introduces noise, wiring instability.

Mitigation: Pre-recorded structured dataset as fallback; edge preprocessing filters noise.

## Layer 2 — Edge Acquisition Layer (ESP32)

The ESP32 microcontroller reads sensor values every second, timestamps each reading, applies optional noise filtering, and transmits a JSON payload via WiFi to the backend API.

ESP32 Responsibilities

• Read MPU6050 via I2C (GPIO 21 = SDA, GPIO 22 = SCL)

• Read DS18B20 temperature via GPIO 4 (with 4.7kΩ pull-up resistor)

• Read ACS712 analog output via GPIO 34

• Add Unix timestamp to each reading

• Serialize to JSON format

• POST JSON to Python backend REST endpoint via WiFi

Sample JSON output from ESP32:

{ "timestamp": "2026-02-26T10:32:14Z", "vibration": 0.42, "temperature": 38.5, "current": 1.8 }

Drawback: WiFi dependency may cause packet loss.

Mitigation: Retry logic in firmware; local SD logging as fallback buffer.

## Layer 3 — Data Ingestion Layer (Python FastAPI)

The backend API receives JSON payloads from the ESP32, validates incoming data, and writes to the raw data store. It is the entry point for the entire software pipeline.

API Endpoints

• POST /ingest — Receives sensor JSON, validates fields, stores raw record

• GET /health — Returns API liveness status

• GET /raw-data — Returns paginated raw sensor records

• POST /label — Allows maintenance team to tag a time-window with fault label

• GET /anomaly-scores — Returns latest anomaly scores for dashboard

Validation Rules

• All fields (timestamp, vibration, temperature, current) must be present

• Values must be within configurable operational ranges

• Timestamp must be valid ISO 8601 format

• Reject malformed payloads with HTTP 400; log for diagnostics

Drawback: Single endpoint can bottleneck under high-frequency polling.

Mitigation: Async FastAPI handlers; configurable ingestion interval (default 1 second).

## Layer 4 — Preprocessing & Feature Engineering Layer

Raw sensor data is inherently noisy. This layer transforms raw readings into stable, ML-ready feature vectors that capture meaningful machine behavior rather than random electrical noise.

Preprocessing Steps

• Step 1 — Moving Average Smoothing: Rolling window (N=10) reduces high-frequency noise

• Step 2 — Outlier Filtering: Values beyond 3σ replaced with rolling median

• Step 3 — Normalization: Min-max scaling to [0,1] range for each sensor channel

• Step 4 — RMS Vibration: Root-Mean-Square of vibration window (captures energy intensity)

• Step 5 — Temperature Gradient: Rate of temperature change per unit time (detects thermal acceleration)

• Step 6 — Current Variance: Statistical variance over sliding window (detects load instability)

• Step 7 — Rolling Statistics: Mean, std, min, max over last 20 readings per channel

Output Feature Vector per timestamp:

| Feature | Formula / Method | Physical Meaning |
| --- | --- | --- |
| rms_vibration | √(mean(v²) over window) | Energy intensity of mechanical vibration |
| temp_gradient | (T_n − T_{n-10}) / 10 | Rate of heat build-up |
| current_variance | Var(I) over last 20 readings | Load fluctuation magnitude |
| vib_rolling_mean | Mean(v) over last 20 | Baseline vibration level |
| vib_rolling_std | Std(v) over last 20 | Vibration instability |
| temp_rolling_max | Max(T) over last 20 | Peak thermal load |

Drawback: Fixed window size may miss slow-developing anomalies.

Mitigation: Multi-scale windows (short + long) used in parallel for richer feature coverage.

## Layer 5 — Time-Series Storage Layer

Processed feature vectors, raw readings, anomaly scores, health scores, and operator labels are all persisted in a structured time-series store. This enables historical analysis, model retraining, and auditable maintenance logs.

Database Schema

• raw_readings: id ¦ timestamp ¦ vibration ¦ temperature ¦ current

• processed_features: id ¦ timestamp ¦ rms_vibration ¦ temp_gradient ¦ current_variance ¦ rolling stats

• anomaly_results: id ¦ timestamp ¦ anomaly_score ¦ anomaly_flag ¦ model_version

• health_scores: id ¦ timestamp ¦ health_score ¦ risk_level

• labels: id ¦ timestamp_start ¦ timestamp_end ¦ fault_type ¦ labeled_by ¦ notes

• retraining_log: id ¦ retrain_date ¦ model_type ¦ accuracy ¦ f1_score

Storage Technology: SQLite for MVP (zero-setup, file-based). Production upgrade path: InfluxDB for high-frequency time-series with Grafana visualization.

Drawback: SQLite may degrade at very high ingestion rates.

Mitigation: Write-ahead logging enabled; InfluxDB path documented for production scale-up.

## Layer 6 — ML Intelligence Engine

This is the cognitive core of GuardX. The intelligence layer follows an evolutionary learning lifecycle — beginning with unsupervised anomaly detection and progressively transitioning to supervised failure classification as labeled data accumulates.

Learning Lifecycle: Unsupervised → Hybrid → Supervised

Phase A — Unsupervised Mode (Initial Deployment)

Used when there is no historical failure data. The system learns what 'normal' looks like and flags deviations.

Phase A Details

• Algorithm: Isolation Forest (scikit-learn)

• Input: Feature vectors from Preprocessing Layer

• Training: Baseline data collected during confirmed normal operation period

• Output: Anomaly score (-1 to 1 scale, normalized to 0–100) + binary anomaly flag

• Trigger: System enters Phase A on first deployment or after factory reset

• Limitation: Cannot identify type of failure — only that deviation exists

Adaptive Threshold Tuning: Initial detection threshold set at 3σ from normal distribution. Maintenance team feedback adjusts sensitivity to reduce false positives over first 2 weeks.

Drawback: May produce false positives during machine warm-up or planned load variation.

Mitigation: Persistence validation — alert only triggers if anomaly persists for 5 consecutive readings. Threshold configurable per machine profile.

Phase B — Hybrid Mode (Data Accumulation Stage)

Triggered automatically when labeled fault events exceed a configurable threshold (default: 50 labeled events). The system now runs both models in parallel.

Phase B Details

• Stage 1 — Anomaly Detection: Isolation Forest still runs to detect any deviation (catches novel anomalies)

• Stage 2 — Failure Classification: Random Forest / XGBoost classifier identifies specific fault type

• Classifier Training: Historical labeled data from operator fault confirmations

• Failure Classes: Bearing Wear ¦ Mechanical Imbalance ¦ Thermal Overload ¦ Electrical Overload ¦ Normal Anomaly

• Output: Anomaly flag + Fault type probability distribution (e.g., 70% Bearing, 20% Imbalance, 10% Normal)

• Trigger: Automated when label count threshold reached; admin can manually trigger

Drawback: Classification accuracy limited by quantity and quality of labeled data.

Mitigation: Human-in-the-loop labeling UI in dashboard; minimum data quality checks before Phase B activation.

Phase C — Fully Supervised Optimization

When labeled dataset is sufficiently large and diverse, the supervised model becomes the primary decision engine. Isolation Forest acts as a safety net for unseen anomaly patterns.

Phase C Details

• Primary Model: XGBoost classifier (higher accuracy on tabular data)

• Safety Net: Isolation Forest retained for out-of-distribution detection

• Model Versioning: Each retrained model saved with version, date, performance metrics

• Continuous Learning: Periodic retraining scheduled (default: weekly if new labels exist)

• Output: Failure type with confidence %, risk timeline estimate, recommended maintenance action

Continuous Learning Loop

This loop is what makes GuardX self-improving over time:

| 1 | Anomaly Detected ML engine flags anomaly, generates alert, logs event to DB |
| --- | --- |

| 2 | Operator Review Maintenance engineer inspects machine, confirms or dismisses alert |
| --- | --- |

| 3 | Fault Labeling Operator tags event in dashboard with fault type (via /label endpoint) |
| --- | --- |

| 4 | Label Stored Labeled record written to labels table in DB with timestamp window |
| --- | --- |

| 5 | Threshold Check System checks if label count exceeds Phase B/C activation threshold |
| --- | --- |

| 6 | Retraining Scheduled retrainer loads labeled data, retrains supervised model |
| --- | --- |

| 7 | Model Deployment New model saved, versioned, deployed to ML engine |
| --- | --- |

## Layer 7 — Health Score & Risk Engine

Converts ML outputs into a single, human-readable Machine Health Score. This bridges the gap between complex ML predictions and simple operational decisions.

Health Score Formula

• Health Score = 100 − (w₁ × rms_anomaly + w₂ × temp_anomaly + w₃ × current_anomaly) × 100

• Default Weights: Vibration (w₁ = 0.50) ¦ Temperature (w₂ = 0.30) ¦ Current (w₃ = 0.20)

• Smoothing: Rolling average over last 10 health scores (prevents sudden jumps)

• Persistence Validation: Score change > 15 points must persist 5 readings before UI update

• Weights are configurable per machine profile via admin settings

Risk Level Classification:

| Health Score | Risk Level | Status Color | Recommended Action |
| --- | --- | --- | --- |
| 80 – 100 | Healthy | Green | Continue normal operation |
| 60 – 79 | Warning | Yellow | Schedule inspection within 48 hours |
| 40 – 59 | Critical | Orange | Immediate inspection; reduce load |
| 0 – 39 | High Risk | Red | Emergency shutdown; do not operate |

## Layer 8 — Alert & Visualization Layer

The Streamlit dashboard is the operator-facing interface. It translates all backend intelligence into real-time, actionable visual information.

Dashboard Panels

• Panel 1 — Live Sensor Feed: Real-time vibration, temperature, current trend lines (auto-refresh every 1 second)

• Panel 2 — Machine Health Gauge: Circular gauge (0–100) with color-coded risk zones

• Panel 3 — Status Indicator: Large color banner (Green/Yellow/Orange/Red) with current status label

• Panel 4 — Anomaly Timeline: Historical anomaly score plotted over time with threshold line

• Panel 5 — Recent Alerts Table: Last 20 alerts with timestamp, severity, and current status

• Panel 6 — Fault Classification Panel: Pie chart of failure probability distribution (Phase B+)

• Panel 7 — Labeling Interface: Date-range selector + fault type dropdown for operator labeling

Alert Trigger Conditions

• Health Score < 60 and persists for ≥ 5 consecutive readings → Warning Alert

• Health Score < 40 → Critical Alert (immediate, no persistence check)

• Anomaly flag = True for ≥ 5 consecutive readings → Anomaly Alert

• Rapid temperature gradient (>2°C/min) → Thermal Alert

• Alert channels: On-screen banner ¦ Optional Telegram bot ¦ Optional SMS via Twilio API

# 4. Hackathon Demo Flow

The demo is structured as a walkthrough of the live system, showing each layer functioning in sequence. The ML model runs on a pre-trained structured dataset for stability; live sensor data passes through the full pipeline from Layer 1 to Layer 8.

| STEP 1 | Physical Setup Introduction Point to motor testbed. Explain sensors and what each measures. Show ESP32 sending data to laptop. |
| --- | --- |

| STEP 2 | Live Sensor Data in Dashboard Open Streamlit dashboard. Sensors streaming live. Show raw vibration, temperature, current graphs updating in real time. |
| --- | --- |

| STEP 3 | Normal Operation Baseline Motor running balanced. Health score shows 90–100 (Green). Demonstrate stable baseline. |
| --- | --- |

| STEP 4 | Trigger Imbalance Fault Attach offset weight to motor shaft. Vibration anomaly score rises. Health score drops to 65 (Yellow). Warning alert triggers. |
| --- | --- |

| STEP 5 | ML Model Explanation Explain Isolation Forest detection. Show anomaly score calculation. Explain transition to supervised classification. |
| --- | --- |

| STEP 6 | Operator Labeling Demo Use dashboard labeling panel to label the triggered anomaly as 'Mechanical Imbalance'. Show label stored in DB. |
| --- | --- |

| STEP 7 | Health Score Deep Dive Explain scoring formula. Show how weights are applied. Demonstrate risk level thresholds. |
| --- | --- |

| STEP 8 | Continuous Learning Roadmap Explain how labeled data triggers Phase B → Phase C. Show evolution from detection to classification. |
| --- | --- |

# 5. Functional Requirements Summary

| Requirement ID | Description | Priority |
| --- | --- | --- |
| FR-01 | Capture vibration, temperature, current at 1Hz minimum | MUST HAVE |
| FR-02 | Transmit JSON payload via WiFi within 2 seconds | MUST HAVE |
| FR-03 | Validate and reject malformed sensor payloads | MUST HAVE |
| FR-04 | Apply moving average and normalization preprocessing | MUST HAVE |
| FR-05 | Compute RMS vibration, temperature gradient, current variance | MUST HAVE |
| FR-06 | Store raw and processed data with timestamps | MUST HAVE |
| FR-07 | Train and run Isolation Forest anomaly detection | MUST HAVE |
| FR-08 | Generate anomaly score and anomaly flag per reading | MUST HAVE |
| FR-09 | Compute Machine Health Score (0–100) | MUST HAVE |
| FR-10 | Classify risk level (Healthy/Warning/Critical/High Risk) | MUST HAVE |
| FR-11 | Display live sensor graphs in dashboard | MUST HAVE |
| FR-12 | Show health score gauge and status indicator | MUST HAVE |
| FR-13 | Trigger alert when Health Score < 60 for 5 readings | MUST HAVE |
| FR-14 | Allow operator to label fault events via dashboard | SHOULD HAVE |
| FR-15 | Transition to supervised classifier when label count threshold met | SHOULD HAVE |
| FR-16 | Send Telegram / SMS alert on critical threshold breach | NICE TO HAVE |
| FR-17 | Support on-premise and cloud deployment modes | NICE TO HAVE |
| FR-18 | Automatic model retraining on schedule | NICE TO HAVE |

# 6. Technology Stack

| Component | Technology / Tool |
| --- | --- |
| Microcontroller | ESP32 DevKit V1 (WiFi + GPIO) |
| Vibration Sensor | MPU6050 (I2C, 3-axis accelerometer) |
| Temperature Sensor | DS18B20 (Digital, 1-Wire) |
| Current Sensor | ACS712 5A module |
| Motor | 12V DC Geared Motor (300–600 RPM) |
| Motor Driver | L298N dual H-bridge module |
| Backend API | Python + FastAPI + Uvicorn |
| Preprocessing | Python + Pandas + NumPy |
| ML Engine (Unsupervised) | scikit-learn Isolation Forest |
| ML Engine (Supervised) | scikit-learn Random Forest / XGBoost |
| Model Persistence | joblib (.pkl files) |
| Database (MVP) | SQLite via SQLAlchemy |
| Database (Production) | InfluxDB + Grafana |
| Dashboard | Streamlit |
| Data Simulation | Python synthetic time-series generator |
| Alert (Optional) | Telegram Bot API / Twilio SMS API |
| Version Control | Git / GitHub |

# 7. Risks & Mitigation Strategies

| Risk | Likelihood | Mitigation | Impact |
| --- | --- | --- | --- |
| Hardware sensor failure during demo | Medium | Pre-recorded backup dataset; software simulation mode toggle in dashboard | High |
| WiFi packet loss / connectivity issues | Medium | Local SD card buffer on ESP32; retry logic in firmware | Medium |
| ML model false positives | High (early phase) | Persistence validation (5 reading window); adaptive threshold tuning | Medium |
| Insufficient labeled data for supervised model | High (early) | Start with unsupervised; manual labeling UI; graduated transition logic | Medium |
| Model drift over time | Low | Weekly retraining schedule; model versioning and rollback capability | High |
| SQLite performance at scale | Low (MVP) | Write-ahead logging; documented InfluxDB migration path | Low |
| Noisy raw sensor data | High | Multi-stage preprocessing; outlier removal; rolling statistics | Medium |

# 8. Build Roadmap (Before 24th Feb)

| Phase | Focus | Deliverables | Status |
| --- | --- | --- | --- |
| Phase 0 | Hardware Prep | Order components; wooden base assembly; initial wiring | Day 1–2 |
| Phase 1 | Data Simulation | Python synthetic CNC data generator (normal + fault patterns) | Day 1–2 |
| Phase 2 | Backend API | FastAPI ingestion server; SQLite schema; validation logic | Day 2–3 |
| Phase 3 | Preprocessing Engine | Moving average, normalization, feature extraction pipeline | Day 3 |
| Phase 4 | Isolation Forest ML | Train on structured dataset; save .pkl; anomaly scoring | Day 3–4 |
| Phase 5 | Health Score Engine | Weighted formula; risk classification; smoothing logic | Day 4 |
| Phase 6 | Streamlit Dashboard | Live graphs, health gauge, alert banner, labeling UI | Day 4–5 |
| Phase 7 | ESP32 Integration | Replace simulator with real sensor stream from ESP32 | Day 5–6 |
| Phase 8 | Demo Rehearsal | End-to-end flow test; imbalance fault demo; Q&A prep | Day 6–7 |

# 9. Future Enhancements (Post-MVP)

Phase 2 — Production Enhancements

• Multi-machine fleet monitoring from a single dashboard

• InfluxDB + Grafana replacement for production-scale time-series storage

• Edge ML deployment (TensorFlow Lite on ESP32) for offline inference

• Cloud deployment with Docker containerization

• Role-based access control (Factory Owner / Engineer / Operator roles)

Phase 3 — Advanced Intelligence

• Remaining Useful Life (RUL) prediction using LSTM time-series models

• Digital Twin integration for virtual machine simulation

• Root Cause Analysis module with explainable AI (SHAP values)

• Automated maintenance scheduling optimizer

• Integration with ERP/MES systems via REST API

Phase 4 — Enterprise & SME Scale-Up

• SaaS offering with per-machine subscription model

• Plug-and-play hardware kit for zero-configuration deployment

• Mobile app for maintenance team alerts and approvals

• Multi-language dashboard support for regional SME adoption

• Industry-standard API compatibility (OPC-UA, MQTT)

# 10. GuardX Value Proposition

GuardX is not just an anomaly detector. It is a self-evolving predictive maintenance framework designed to democratize industrial AI for SMEs.

| Traditional Approach | GuardX Approach |
| --- | --- |
| Reactive maintenance (fix after failure) | Predictive maintenance (prevent failure) |
| Expensive proprietary software | 100% open-source, no licensing cost |
| Complex setup requiring IT expertise | Modular, plug-and-play architecture |
| Static detection rules | Self-learning ML system that improves over time |
| Enterprise-only access | Designed specifically for SME affordability |
| Dashboard requires ML knowledge | Machine Health Score for any operator |
| Single-machine, monolithic systems | Scalable to multi-machine fleet monitoring |

"Predictive maintenance should not be a luxury. GuardX makes it accessible."

GuardX v1.0 — TN Impact 2026 | TNI26026 | App Flow Document