PRODUCT REQUIREMENTS DOCUMENT (PRD)

Product Name: GuardX

Version: 1.0 (Hackathon MVP)

Date: Feb 2026

1️⃣ Product Overview

1.1 Purpose

GuardX is a low-cost, open-source predictive maintenance system designed specifically for Small and Medium Enterprises (SMEs) operating CNC machines.

The product enables real-time machine monitoring, anomaly detection, and progressive evolution from unsupervised anomaly detection to supervised fault classification.

1.2 Problem Statement

CNC machines are critical production assets. Unexpected breakdowns cause:

- Production downtime

- Financial losses

- Delayed deliveries

- Reduced machine life

Existing predictive maintenance solutions:

- Are expensive

- Require proprietary hardware

- Not customizable

- Not SME-friendly

There is a gap for:

Affordable, modular, open-source predictive maintenance system.

1.3 Vision

Make predictive maintenance accessible, scalable, and self-improving for SMEs.

2️⃣ Goals & Objectives

2.1 Primary Goals

- Real-time CNC health monitoring

- Early anomaly detection

- Gradual transition to failure prediction

- Low-cost deployment

- Open-source architecture

2.2 Success Metrics

- Detect ≥ 80% abnormal patterns

- Reduce simulated downtime by ≥ 30%

- Health score accuracy correlation with fault events

- Real-time latency < 2 seconds

3️⃣ Target Users

3.1 Primary Users

- SME factory owners

- Maintenance engineers

- Production supervisors

3.2 Secondary Users

- Industrial IoT integrators

- Technical colleges

- Research labs

4️⃣ System Architecture Overview

GuardX consists of 8 layers:

- Physical Machine Layer

- Edge Acquisition Layer

- Data Ingestion Layer

- Preprocessing & Feature Engineering

- Time-Series Storage

- Intelligence Engine

- Health & Risk Engine

- Alert & Visualization Layer

5️⃣ Functional Requirements

5.1 Layer 1 – Physical Machine Layer

FR-1

System shall collect:

- Vibration data

- Temperature data

- Current (load) data

FR-2

System shall simulate:

- Imbalance

- Load variation

- Gradual heating

5.2 Layer 2 – Edge Acquisition Layer (ESP32)

FR-3

System shall:

- Capture sensor values every configurable interval (default 1 sec)

FR-4

System shall:

- Send JSON data via WiFi

Example:

{
  "timestamp": "...",
  "vibration": 0.42,
  "temperature": 38.5,
  "current": 1.8
}

5.3 Layer 3 – Data Ingestion Layer

FR-5

System shall provide REST endpoint for receiving sensor data.

FR-6

System shall validate:

- Missing values

- Out-of-range values

FR-7

System shall log raw data.

5.4 Layer 4 – Preprocessing Layer

FR-8

System shall perform:

- Moving average smoothing

- Outlier filtering

- Normalization

FR-9

System shall compute:

- RMS vibration

- Temperature gradient

- Current variance

- Rolling window statistics

5.5 Layer 5 – Time-Series Storage

FR-10

System shall store:

- Raw data

- Processed features

- Anomaly score

- Health score

- Labels

FR-11

System shall support:

- Historical querying

- Model retraining

5.6 Layer 6 – Intelligence Engine

Phase A – Unsupervised Mode

FR-12

System shall use Isolation Forest.

FR-13

System shall output:

- Anomaly score

- Anomaly flag

Limitation:
No failure classification.

Phase B – Hybrid Mode

Trigger:
When labeled maintenance events > threshold.

FR-14

System shall run:

- Unsupervised anomaly detection

- Supervised classification (Random Forest / XGBoost)

FR-15

System shall output:

- Failure probability

- Failure class

Phase C – Fully Supervised Mode

FR-16

Primary decision shall rely on supervised model.

FR-17

Unsupervised model shall act as fallback anomaly detector.

5.7 Layer 7 – Health & Risk Engine

FR-18

System shall compute Health Score:

Health = 100 − Weighted Risk Score

FR-19

System shall classify:

80–100 → Healthy
60–80 → Warning
40–60 → Critical
<40 → High Risk

FR-20

System shall smooth health score using rolling average.

5.8 Layer 8 – Visualization & Alerts

FR-21

System shall provide dashboard with:

- Live sensor graphs

- Health gauge

- Status indicator

- Risk trends

FR-22

System shall trigger alerts when:

- Health < threshold

- Persistent anomaly detected

Optional:

- Telegram notification

- SMS alert

6️⃣ Non-Functional Requirements

6.1 Performance

- Real-time processing < 2 sec delay

- Dashboard refresh rate < 1 sec

6.2 Scalability

- Support multiple machines

- Modular sensor integration

6.3 Reliability

- Fault-tolerant ingestion

- Graceful failure handling

6.4 Security

- Basic API validation

- Local deployment option

7️⃣ Data Flow

- Sensor data generated

- ESP32 transmits JSON

- Backend stores raw data

- Preprocessing engine extracts features

- ML engine computes anomaly score

- Health engine computes health

- Dashboard updates

- Alerts triggered

- Labeled data stored

- Periodic retraining

8️⃣ Risks & Mitigation

| Risk | Mitigation |
| --- | --- |
| No real CNC data | Use simulated + incremental real testbed data |
| Insufficient labels | Begin with unsupervised learning |
| Sensor noise | Apply filtering & smoothing |
| Model drift | Periodic retraining |
| Hardware failure | Modular replaceable components |

9️⃣ MVP Scope (Hackathon Version)

Includes:

- 1 machine monitoring

- Unsupervised anomaly detection

- Health score engine

- Live dashboard

- Manual labeling option

Excludes:

- Multi-machine scaling

- Cloud deployment

- Advanced root cause analytics

🔟 Future Enhancements

- Edge ML deployment

- Cloud-based fleet monitoring

- Digital twin integration

- Remaining Useful Life (RUL) prediction

- Maintenance scheduling optimizer

🧠 Final Product Positioning Statement

GuardX is not just an anomaly detector.
It is a self-evolving predictive maintenance framework designed to democratize industrial AI for SMEs.